# Hercules Terminator Family Technical Information

Release 1.00 – March 15th, 1996

## Table Of Contents

Hercules Terminator Family Technical Information

Table Of Contents

Introduction

Disclaimers And Legal Notices

Contact Information

Model Numbers

Memory and I/O Address Locations

Models GT501, GT1002, GT2020, GT2040, GT3020, GT3040

Models GT801, GT1202, GT2220, GT2240, GT3220, GT3240, T2211, T2212, T2212EDO, T2212M, T2212MEDO, T2312, T2312M

A Note On COM4:

Analog Video Connector Pinouts

All Models

Feature Connector Pin Assignment

All Models

Upgrading Memory

Model GT1001

Model GT1201

Models GT3020, GT3220

Model T2211

Terminator Display Modes

Terminator 64/Video, Terminator 64/3D

Terminator 32/DRAM, Terminator 64/DRAM, Terminator 64/VRAM, Terminator Professional

SETCRT

Master Mode Table

## Introduction

This document contains technical information on the Hercules Terminator family of graphics, multimedia and 3D accelerators.  It’s designed to be used as an adjunct to our printed installation guides.  You may find this information helpful if you’re programming a Hercules Terminator product, troubleshooting a problem, or if you simply would like more information on the ins and outs of the Terminator product family.

We’re always looking for ways to improve it.  Any information we’re missing?  Let us know.

## Disclaimers And Legal Notices

Hercules Computer Technology, Inc. makes every effort to ensure that this document is accurate.  However, because we are always striving to improve our products, we are unable to guarantee the accuracy of this document after the date of publication, and we disclaim liability for any changes, errors or omissions.  No reproduction of this document, in any form, is allowed without the written permission of Hercules Computer Technology, Inc.

Copyright © 1996, Hercules Computer Technology, Inc., 3839 Spinnaker Court, Fremont, CA 94538, 510-623-6030.  Hercules is a registered trademark and Hercules Terminator and Hercules Terminator Professional are trademarks of Hercules Computer Technology, Inc.

## Contact Information

Here’s how to reach Hercules:

|  | United States | Germany |
| --- | --- | --- |
| Telephone: | (800) 323-0601 or (510) 623-6050 | +49-89-8989-0573 |
| Fax: | - 623-4215 Fax-back service: (800) 711-HERC | +49-89-8989-0585 |
| BBS: | (510) 623-7449 | Analog: +49-89-8989-0576 ISDN: +49-89-8989-0234 |
| Mail: | Hercules Computer Technology 3839 Spinnaker Court Fremont, CA 94538 | Hercules Computer Technology Bussarstr. 5 D-82166 Gräfelfing München |
| CompuServe: | GO HERCULES |  |
| World Wide Web: | www.hercules.com |  |
| FTP: | ftp.hercules.com |  |
| e-mail: | support@hercules.com |  |

Our U.S. support hotline is open from 6:00 AM to 6:00 PM Pacific Time Monday through Friday.  Our support hotline in Germany is open from 10:00AM to 4:00PM MEZ Monday through Friday.  Our support representatives in Germany speak German and English.

## Model Numbers

| Model Number | Product | Accelerator Chip | Memory Configuration | Bus Type |
| --- | --- | --- | --- | --- |
| GT1001 | Terminator 64/DRAM | S3 Trio64 | 1MB DRAM | VL-Bus |
| GT1002 | Terminator 64/DRAM | S3 Trio64 | 2MB DRAM | VL-Bus |
| GT1201 | Terminator 64/DRRAM | S3 Trio64 | 1MB DRAM | PCI |
| GT1202 | Terminator 64/DRAM | S3 Trio64 | 2MB DRAM | PCI |
| GT2020 | Terminator 64/VRAM | S3 Vision964 | 2MB VRAM | VL-Bus |
| GT2040 | Terminator 64/VRAM | S3 Vision964 | 4MB VRAM | VL-Bus |
| GT2220 | Terminator 64/VRAM | S3 Vision964 | 2MB VRAM | PCI |
| GT2240 | Terminator 64/VRAM | S3 Vision964 | 4MB VRAM | PCI |
| GT3020 | Terminator Professional | S3 Vision968 | 2MB VRAM | VL-Bus |
| GT3040 | Terminator Professional | S3 Vision968 | 4MB VRAM | VL-Bus |
| GT3220 | Terminator Professional | S3 Vision968 | 2MB VRAM | PCI |
| GT3240 | Terminator Professional | S3 Vision968 | 4MB VRAM | PCI |
| GT501 | Terminator 32/DRAM | S3 Trio32 | 1MB DRAM | VL-Bus |
| GT801 | Terminator 32/DRAM | S3 Trio32 | 1MB DRAM | PCI |
| T2211 | Terminator 64/Video | S3 Trio64V+ | 1MB DRAM | PCI |
| T2212 | Terminator 64/Video | S3 Trio64V+ | 2MB DRAM | PCI |
| T2212EDO | Terminator 64/Video | S3 Trio64V+ | 2MB single-cycle EDO (1EDO) DRAM | PCI |
| T2212M | Terminator 64/Video with MPEG Decompression Module (MDM) | S3 Trio64V+ | 2MB DRAM | PCI |
| T2212MEDO | Terminator 64/Video with MDM | S3 Trio64V+ | 2MB 1EDO DRAM | PCI |
| T2312 | Terminator 64/3D | S3 ViRGE | 2MB 1EDO DRAM | PCI |
| T2312M | Terminator 64/3D with MDM | S3 ViRGE | 2MB 1EDO DRAM | PCI |

## Memory and I/O Address Locations

### Models GT501, GT1002, GT2020, GT2040, GT3020, GT3040

Display memory addresses:

A0000h–BFFFFh

ROM BIOS:

C0000h–C7FFFh

I/O locations:

102, 2EA, 3B4, 3B5, 3B8–3BB, 3BF, 3C0–3CF, 3D4, 3D5, 3D8–3DC, 42E8, 46E8, 4AE8, 82E8, 86E8, 8AE8, 8EE8, 92E8, 96E8, 9AE8, 9EE8, A2E8, A6E8, AAE8, AEE8, E2E8, E2EA

### Models GT801, GT1202, GT2220, GT2240, GT3220, GT3240, T2211, T2212, T2212EDO, T2212M, T2212MEDO, T2312, T2312M

Display memory addresses:

A0000h–BFFFFh

ROM BIOS:

Relocatable by system.

I/O locations:

102, 2EA, 3B4, 3B5, 3B8–3BB, 3BF, 3C0–3CF, 3D4, 3D5, 3D8–3DC, 42E8, 46E8, 4AE8, 82E8, 86E8, 8AE8, 8EE8, 92E8, 96E8, 9AE8, 9EE8, A2E8, A6E8, AAE8, AEE8, E2E8, E2EA

### A Note On COM4:

All Terminator cards based on S3’s Vision, Trio and ViRGE series chips will conflict with most systems using the standard COM4: address of 2E8.

The S3 controllers, having been based on IBM's original 8514 design, use several 16-bit addresses at x2E8 (where x equals 4, 8, 9, A or E).

These DO NOT conflict with the standard 12-bit COM4: address of 2E8.

The problem is caused by the serial controllers only decoding the first 12-bits of the I/O port address.  These serial controllers are unable to distinguish between reads/writes to 2E8 and reads/writes to 42E8 (or 82E8, 92E8, etc.).  This causes the conflict between the S3 Vision/Trio/ViRGE accelerator and the COM4: port.

There is no workaround or fix for this problem, other than disabling COM4:

## Analog Video Connector Pinouts

### All Models

| Pin # | Description |
| --- | --- |
| 1 | Red |
| 2 | Green |
| 3 | Blue |
| 4 | Monitor ID Bit 2 |
| 5 | Ground |
| 6 | Ground |
| 7 | Ground |
| 8 | Ground |
| 9 | Not Used |
| 10 | Ground |
| 11 | Not Used |
| 12 | Monitor ID Bit 1 |
| 13 | Horizontal Sync |
| 14 | Vertical Sync |
| 15 | Not Used |

## Feature Connector Pin Assignment

The feature connector found on the Terminator delivers pixel data from the graphics accelerator chip onboard to an external device.  All Terminator models ship with the VGA Feature Connector disabled.  To enable the feature connector, you’ll need to run a utility program called S3SNOOP.COM.

To run FCON, follow these steps:

- Change to the directory containing the Terminator software.

- Run the S3SNOOP.EXE program.

This will enable the VGA feature connector.  Due to the design of the S3 processors used on the Terminator, enabling the VGA feature connector will slow down the performance of your card.  We suggest that you enable the VGA feature connector only when you need to use it.

### All Models

| Pin | Description |
| --- | --- |
| 1 | Pixel Data 0 |
| 2 | Pixel Data 1 |
| 3 | Pixel Data 2 |
| 4 | Pixel Data 3 |
| 5 | Pixel Data 4 |
| 6 | Pixel Data 5 |
| 7 | Pixel Data 6 |
| 8 | Pixel Data 7 |
| 9 | Pixel Clock |
| 10 | Blank Signal |
| 11 | Horizontal Sync |
| 12 | Vertical Sync |
| 17 | Enable External Pixel Data † |
| 18 | Enable External Sync ‡ |
| 19 | Enable External Pixel § |
| 13–16, 21–24 | Ground |
| 20, 25, 26 | Not Used |

†	When pin 17 is pulled low, Pixel Data from Feature Connector is disabled.

‡	When pin 18 is pulled low, Sync and Blank signals from Feature Connector are disabled.

§	When pin 19 is pulled low, Pixel Clock from Feature Connector is disabled

## Upgrading Memory

Certain Hercules products may be upgraded with additional memory.  More memory provides access to more display modes and color depths.

Important: when adding memory, it’s best to match the memory brand and speed of the installed memory exactly.  When this is not possible, selecting one of the memory types listed below for your product may work.  However, by providing this information, we are not making a guarantee that any memory you add to your Terminator product will work correctly, nor are we guaranteeing the availability of any particular memory type.  When in doubt, consult your reseller.

### Model GT1001

Model GT1001 uses two types of DRAM:

Models which use 256Kx8 SOJ memory will have their first megabyte populated with eight chips.  Approved memory types for expanding to 2MB are Mosel V53C8258HK45 and Mosel V53C8256HK60.

Models which use 256Kx16 SOJ memory will have their first megabyte populated with four chips.  Approved memory types for expanding to 2MB are NEC 424260-60R, TI 245160D2 H Japan-70, and NEC 42460-70.

### Model GT1201

Model GT1201 uses two types of DRAM:

Models which use 256Kx8 SOJ memory will have their first megabyte populated with eight chips.  Approved memory types for expanding to 2MB are Mosel V53C8258HK45 and Mosel V53C8256HK60.

Models which use 256Kx16 SOJ memory will have their first megabyte populated with four chips.  Approved memory types for expanding to 2MB are Micron MT 4C16257DJ-7, Hyundai HY514260BJC-60, Hyundai HY514260BJC-70, NPN 514260J-60, Siemens HYB514171BJ-60, and NEC 42S4260-70.

### Models GT3020, GT3220

Note: not all GT3020 and GT3220 units are upgradeable.  We are unable to exchange non-upgradeable units for upgradeable units.

These models use VR256Kx8 SOJ memory.  Approved memory types include Samsung KM428C256J-80, Samsung KM428C256J-70, Hyundai HY528256Y JC-60, and Hyundai HY528256Y JC-70.

### Model T2211

This model uses 256Kx16 SOJ memory.  Approved memory types include Hyundai HY514260BJC-60, Mosel V53C16256HK50, NPN 514260J-60, Silicon Magic SM51C256K16-60, and Samsung KM416C256BJ-60.

## Terminator Display Modes

### Terminator 64/Video, Terminator 64/3D

| Resolution | 256 colors (8 bits per pixel) | 65,536 colors (16 bits per pixel) | 16.7M colors (24 bits per pixel) |
| --- | --- | --- | --- |
| 640x480 | 60, 72, 75, 90, 120Hz |  |  |
| 800x600 | 60, 72, 75, 90, 120Hz | 60, 72, 75, 90Hz | 60, 72, 75, 90Hz |
| 1024x768 | 60, 72, 75, 90, 120Hz | 60, 72, 75, 90Hz |  |
| 1280x1024 | 60, 72, 75Hz |  |  |
| 1600x1200 | 86Hz interlaced |  |  |

The 1024x768x16bpp, 1280x1024x8bpp, and 1600x1200x8bpp modes are available only on cards with 2MB of memory.

### Terminator 32/DRAM, Terminator 64/DRAM, Terminator 64/VRAM, Terminator Professional

### SETCRT

This utility program sets up the Graphite Terminator to work with your monitor.  By default, INSTALL has configured SETCRT to use the monitor timings specified in MYSCREEN.CRT, created when you selected your monitor during the installation process.  You can also run SETCRT with command line arguments to try timings different from those specified in MYSCREEN.CRT.

To run SETCRT to configure Graphite Terminator for your monitor, follow these steps:

1.	Change to the directory containing the Graphite Terminator software by typing CD \HERCULES

There are two ways to run SETCRT to configure Graphite Terminator for your monitor: with a .CRT file or with a specific combination of monitor timings you specify.

#### To run SETCRT with a .CRT file

2a.	Type SETCRT crt_filename  [Enter] at the DOS prompt.  Using the default .CRT file created by INSTALL, you would type the following:

SETCRT MYSCREEN.CRT[Enter]

#### To run SETCRT with specific timings

2b.	You can use SETCRT with the -n option to specify any combination of refresh rates you choose.  The syntax is:

SETCRT -n R1 R2 R3 R4 R5 R6

where R1 is the value for 640x480;

where R2 is the value for 800x600;

where R3 is the value for 1024x768;

where R4 is the value for 1152x864;

where R5 is the value for 1280x1024;

where R6 is the value for 1600x1200.

The following table lists the screen refresh rates available at each resolution.

NOTE:  DO NOT choose a refresh rate higher than your monitor can support.  If you do, you may permanently damage your monitor.  If you are unsure of what refresh rates are supported, please refer to the documentation on your monitor or contact the manufacturer.  Please locate this information before attempting to use SETCRT.

|  |  | Terminator 64/DRAM |  | Terminator 64/VRAM |  | Terminator Professional |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Resolution | Value | Horiz (kHz) | Vert (Hz) | Horiz (kHz) | Vert (Hz) | Horiz (kHz) | Vert (Hz) |
| 640x480 | 0 1 2 3 4 5 | 31.50 38.00 38.00 48.00 64.50 — | 60 72 75 90 120 — | 31.50 38.00 38.00 48.00 64.50 79.50 | 60 72 75 90 120 150 | 31.50 38.00 38.00 48.00 64.50 79.50 | 60 72 75 90 120 150 |
| 800x600 | 0 1 2 3 4 5 6 | 35.50 38.00 48.00 48.00 56.50 78.00 — | 56 60 72 75 90 120 — | 35.50 38.00 48.00 48.00 56.50 78.00 100.00 | 56 60 72 75 90 120 150 | 35.50 38.00 48.00 48.00 56.50 78.00 100.00 | 56 60 72 75 90 120 150 |
| 1024x768 | 0 1 2 3 4 | 35.5 48.00 56.00 60.00 74.00 | 87(I) 60 70 75 90 | 48.00 56.00 60.00 74.00 100.00 | 60 70 75 90 120 | 48.00 56.00 60.00 74.00 100.00 | 60 70 75 90 120 |
| 1152x864 | 0 | 55.00 | 60 | 55.00 | 60 | 55.00 | 60 |
| 1280x1024 | 0 1 2 3 4 5 | 48.00 64.00 78.00 80.00 — — | 87(I) 60 72 75 — — | 48.00 64.00 78.00 80.00 92.00 — | 87(I) 60 72 75 85 — | 48.00 64.00 78.00 80.00 92.00 108.00 | 87(I) 60 72 75 85 100 |
| 1600x1200 | 0 1 2 | 62.00 — — | 98(I) — — | 76.50 — — | 60 — — | 76.50 90.00 96.00 | 60 72 75 |

- Interlaced modes

NOTE: Refresh rates for 16- and 24-bit display modes may be lower than values shown in these tables.   Consult the MONITORS.TXT file for a complete listing of modes and refresh rates.

## Master Mode Table

The following table lists the video modes available from the different Graphite Terminator models.  Please note that not all Terminator models are capable of all video modes.

Also, the refresh rates, resolutions and pixel depths listed are driver dependent and may not be available in all operating systems or applications.  Please see the Installation Guide for a list of modes and refresh rates available for each product.

| Mode (in Hex) | Type of Mode | Screen Resolution | Number of Colors | Max. Characters | Buffer Start | Character Size | Pages | Video Clock (MHz) | Horiz. Freq. (kHz) | Vert. Freq. (Hz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Text | 320 x 200 | 16 | 40 x 25 | B8000 | 8 x 8 | 8 | 28.322 | 31.50 | 70.00 |
| 0* | Text | 320 x 350 | 16 | 40 x 25 | B8000 | 8 x 14 | 8 | 28.322 | 31.50 | 70.00 |
| 0** | Text | 360 x 400 | 16 | 40 x 25 | B8000 | 9 x 16 | 8 | 28.322 | 31.50 | 70.00 |
| 1 | Text | 320 x 200 | 16 | 40 x 25 | B8000 | 8 x 8 | 8 | 28.322 | 31.50 | 70.00 |
| 1* | Text | 320 x 350 | 16 | 40 x 25 | B8000 | 8 x 14 | 8 | 28.322 | 31.50 | 70.00 |
| 1** | Text | 360 x 400 | 16 | 40 x 25 | B8000 | 9 x 16 | 8 | 28.322 | 31.50 | 70.00 |
| 2 | Text | 640 x 200 | 16 | 80 x 25 | B8000 | 8 x 8 | 8 | 28.322 | 31.50 | 70.00 |
| 2* | Text | 640 x 350 | 16 | 80 x 25 | B8000 | 8 x 14 | 8 | 28.322 | 31.50 | 70.00 |
| 2** | Text | 720 x 400 | 16 | 80 x 25 | B8000 | 9 x 16 | 8 | 28.322 | 31.50 | 70.00 |
| 3 | Text | 640 x 200 | 16 | 80 x 25 | B8000 | 8 x 8 | 8 | 28.322 | 31.50 | 70.00 |
| 3* | Text | 640 x 350 | 16 | 80 x 25 | B8000 | 8 x 14 | 8 | 28.322 | 31.50 | 70.00 |
| 3** | Text | 720 x 400 | 16 | 80 x 25 | B8000 | 9 x 16 | 8 | 28.322 | 31.50 | 70.00 |
| 4 | Graphics | 320 x 200 | 4 | 40 x 25 | B8000 | 8 x 8 | 1 | 25.175 | 31.50 | 70.00 |
| 5 | Graphics | 320 x 200 | 4 | 40 x 25 | B8000 | 8 x 8 | 1 | 25.175 | 31.50 | 70.00 |
| 6 | Graphics | 640 x 200 | 2 | 80 x 25 | B8000 | 8 x 8 | 1 | 25.175 | 31.50 | 70.00 |
| 7 | Text | 720 x 350 | Mono | 80 x 25 | B0000 | 9 x 14 | 8 | 28.322 | 31.50 | 70.00 |
| 7** | Text | 720 x 400 | Mono | 80 x 25 | B0000 | 9 x 16 | 8 | 28.322 | 31.50 | 70.00 |
| D | Graphics | 320 x 200 | 16 | 40 x 25 | A0000 | 8 x 8 | 8 | 25.175 | 31.50 | 70.00 |
| E | Graphics | 640 x 200 | 16 | 80 x 25 | A0000 | 8 x 8 | 4 | 25.175 | 31.50 | 70.00 |
| F | Graphics | 640 x 350 | Mono | 80 x 25 | A0000 | 8 x 14 | 2 | 25.175 | 31.50 | 70.00 |
| 10 | Graphics | 640 x 350 | 16 | 80 x 25 | A0000 | 8 x 14 | 2 | 25.175 | 31.50 | 70.00 |
| 11 | Graphics | 640 x 480 | 2 | 80 x 30 | A0000 | 8 x 16 | 1 | 25.175 | 31.50 | 60.00 |
| 11 | Graphics | 640 x 480 | 2 | 80 x 30 | A0000 | 8 x 16 | 1 | 32.514 | 38.70 | 72.70 |
| 12 | Graphics | 640 x 480 | 16 | 80 x 30 | A0000 | 8 x 16 | 1 | 25.175 | 31.50 | 60.00 |
| 12 | Graphics | 640 x 480 | 16 | 80 x 30 | A0000 | 8 x 16 | 1 | 32.514 | 38.70 | 72.70 |
| 12 | Graphics | 640 x 480 | 16 | 80 x 30 | A0000 | 8 x 16 | 1 | 40.000 | 48.40 | 90.00 |
| 13 | Graphics | 320 x 200 | 256 | 40 x 25 | A0000 | 8 x 8 | 1 | 25.175 | 31.50 | 70.00 |
| 100 | Graphics | 640 x400 | 256 | 80 x 60 | A0000 | 8 x 8 | 1 | 25.25 | 31.50 | 70.00 |
| 101 | Graphics | 640 x 480 | 256 | 80 x 30 | A0000 | 8 x 16 | 1 | 25.25 | 31.50 | 60.00 |
| 101 | Graphics | 640 x 480 | 256 | 80 x 30 | A0000 | 8 x 16 | 1 | 31.50 | 38.00 | 72.00 |
| 101 | Graphics | 640 x 480 | 256 | 80 x 30 | A0000 | 8 x 16 | 1 | 31.50 | 38.00 | 75.00 |
| 101 | Graphics | 640 x 480 | 256 | 80 x 30 | A0000 | 8 x 16 | 1 | 40.00 | 48.00 | 90.00 |
| 101 | Graphics | 640 x 480 | 256 | 80 x 30 | A0000 | 8 x 16 | 1 | 53.00 | 64.50 | 120.00 |
| 101 | Graphics | 640 x 480 | 256 | 80 x 30 | A0000 | 8 x 16 | 1 | 75.00 | 79.50 | 150.00 |
| 102 | Graphics | 800 x 600 | 16 | 100 x 37 | A0000 | 8 x 16 | 1 | 36.00 | 35.50 | 56.00 |
| 102 | Graphics | 800 x 600 | 16 | 100 x 37 | A0000 | 8 x 16 | 1 | 40.00 | 38.00 | 60.00 |
| 102 | Graphics | 800 x 600 | 16 | 100 x 37 | A0000 | 8 x 16 | 1 | 50.00 | 48.00 | 72.00 |
| 102 | Graphics | 800 x 600 | 16 | 100 x 37 | A0000 | 8 x 16 | 1 | 49.50 | 48.00 | 75.00 |
| 103 | Graphics | 800 x 600 | 256 | 100 x 37 | A0000 | 8 x 16 | 1 | 36.00 | 35.50 | 56.00 |
| 103 | Graphics | 800 x 600 | 256 | 100 x 37 | A0000 | 8 x 16 | 1 | 40.00 | 38.00 | 60.00 |
| 103 | Graphics | 800 x 600 | 256 | 100 x 37 | A0000 | 8 x 16 | 1 | 50.00 | 48.00 | 72.00 |
| 103 | Graphics | 800 x 600 | 256 | 100 x 37 | A0000 | 8 x 16 | 1 | 49.500 | 48.00 | 75.00 |
| 103 | Graphics | 800 x 600 | 256 | 100 x 37 | A0000 | 8 x 16 | 1 | 58.00 | 56.50 | 90.00 |
| 103 | Graphics | 800 x 600 | 256 | 100 x 37 | A0000 | 8 x 16 | 1 | 45.00*2 | 78.00 | 120.00 |
| 103 | Graphics | 800 x 600 | 256 | 100 x 37 | A0000 | 8 x 16 | 1 | 65.00*2 | 100.00 | 150.00 |
| 104 | Graphics | 1024 x 768 | 16 | 128 x 48 | A0000 | 8 x 16 | 1 | 65.00 | 48.00 | 60.00 |
| 104 | Graphics | 1024 x 768 | 16 | 128 x 48 | A0000 | 8 x 16 | 1 | 75.00 | 56.00 | 70.00 |
| 104 | Graphics | 1024 x 768 | 16 | 128 x 48 | A0000 | 8 x 16 | 1 | 80.00 | 60.00 | 75.00 |
| 105 | Graphics | 1024 x 768 | 256 | 128 x 48 | A0000 | 8 x 16 | 1 | 65.00 | 48.00 | 60.00 |
| 105 | Graphics | 1024 x 768 | 256 | 128 x 48 | A0000 | 8 x 16 | 1 | 75.00 | 56.00 | 70.00 |
| 105 | Graphics | 1024 x 768 | 256 | 128 x 48 | A0000 | 8 x 16 | 1 | 80.00 | 60.00 | 75.00 |
| 105 | Graphics | 1024 x 768 | 256 | 128 x 48 | A0000 | 8 x 16 | 1 | 52.50*2 | 74.00 | 90.00 |
| 105 | Graphics | 1024 x 768 | 256 | 128 x 48 | A0000 | 8 x 16 | 1 | 85.00*2 | 100.00 | 120.00 |
| 106 | Graphics | 1280 x 1024 | 16 | 160 x 64 | A0000 | 8 x 16 | 1 | 40.00*2 | 48.00 | 87.00(I) |
| 107 | Graphics | 1280 x 1024 | 256 | 160 x 64 | A0000 | 8 x 16 | 1 | 40.00*2 | 48.00 | 87.00(I) |
| 107 | Graphics | 1280 x 1024 | 256 | 160 x 64 | A0000 | 8 x 16 | 1 | 55.00*2 | 64.00 | 60.00 |
| 107 | Graphics | 1280 x 1024 | 256 | 160 x 64 | A0000 | 8 x 16 | 1 | 65.00*2 | 78.00 | 72.50 |
| 107 | Graphics | 1280 x 1024 | 256 | 160 x 64 | A0000 | 8 x 16 | 1 | 68.00*2 | 80.00 | 75.50 |
| 107 | Graphics | 1280 x 1024 | 256 | 160 x 64 | A0000 | 8 x 16 | 1 | 85.00*2 | 92.00 | 85.00 |
| 107 | Graphics | 1280 x 1024 | 256 | 160 x 64 | A0000 | 8 x 16 | 1 | 104.00 | 108.00 | 100.00 |
| 109 | Text | 1056 x 400 | 16 | 132 x 25 | B8000 | 8 x 16 | 4 | 40.00 | 31.50 | 70.00 |
| 10A | Text | 1056 x 387 | 16 | 132 x 43 | B8000 | 8 x 9 | 2 | 40.00 | 31.50 | 70.00 |
| 110 | Graphics | 640 x 480 | 32,768 | 80 x 30 | A0000 | 8 x 16 | 1 | 25.25 | 31.50 | 60.00 |
| 110 | Graphics | 640 x 480 | 32,768 | 80 x 30 | A0000 | 8 x 16 | 1 | 31.50 | 38.00 | 72.00 |
| 110 | Graphics | 640 x 480 | 32,768 | 80 x 30 | A0000 | 8 x 16 | 1 | 31.50 | 38.00 | 75.00 |
| 110 | Graphics | 640 x 480 | 32,768 | 80 x 30 | A0000 | 8 x 16 | 1 | 40.00 | 48.00 | 90.00 |
| 110 | Graphics | 640 x 480 | 32,768 | 80 x 30 | A0000 | 8 x 16 | 1 | 53.00 | 64.50 | 120.00 |
| 110 | Graphics | 640 x 480 | 32,768 | 80 x 30 | A0000 | 8 x 16 | 1 | 75.00 | 79.50 | 150.00 |
| 111 | Graphics | 640 x 480 | 65,536 | 80 x 30 | A0000 | 8 x 16 | 1 | 25.25 | 31.50 | 60.00 |
| 111 | Graphics | 640 x 480 | 65,536 | 80 x 30 | A0000 | 8 x 16 | 1 | 31.50 | 38.00 | 72.00 |
| 111 | Graphics | 640 x 480 | 65,536 | 80 x 30 | A0000 | 8 x 16 | 1 | 31.50 | 38.00 | 75.00 |
| 111 | Graphics | 640 x 480 | 65,536 | 80 x 30 | A0000 | 8 x 16 | 1 | 40.00 | 48.00 | 90.00 |
| 111 | Graphics | 640 x 480 | 65,536 | 80 x 30 | A0000 | 8 x 16 | 1 | 53.00 | 64.50 | 120.00 |
| 111 | Graphics | 640 x 480 | 65,536 | 80 x 30 | A0000 | 8 x 16 | 1 | 75.00 | 79.50 | 150.00 |
| 112 | Graphics | 640 x 480 | 16.7M | 80 x 30 | A0000 | 8 x 16 | 1 | 25.25 | 31.50 | 60.00 |
| 112 | Graphics | 640 x 480 | 16.7M | 80 x 30 | A0000 | 8 x 16 | 1 | 31.50 | 38.00 | 72.00 |
| 112 | Graphics | 640 x 480 | 16.7M | 80 x 30 | A0000 | 8 x 16 | 1 | 31.50 | 38.00 | 75.00 |
| 112 | Graphics | 640 x 480 | 16.7M | 80 x 30 | A0000 | 8 x 16 | 1 | 40.00 | 48.00 | 90.00 |
| 112 | Graphics | 640 x 480 | 16.7M | 80 x 30 | A0000 | 8 x 16 | 1 | 53.00 | 64.50 | 120.00 |
| 112 | Graphics | 640 x 480 | 16.7M | 80 x 30 | A0000 | 8 x 16 | 1 | 75.00 | 79.50 | 150.00 |
| 113 | Graphics | 800 x 600 | 32,768 | 100 x 37 | A0000 | 8 x 16 | 1 | 36.00 | 35.50 | 56.00 |
| 113 | Graphics | 800 x 600 | 32,768 | 100 x 37 | A0000 | 8 x 16 | 1 | 40.00 | 38.00 | 60.00 |
| 113 | Graphics | 800 x 600 | 32,768 | 100 x 37 | A0000 | 8 x 16 | 1 | 50.00 | 48.00 | 72.00 |
| 113 | Graphics | 800 x 600 | 32,768 | 100 x 37 | A0000 | 8 x 16 | 1 | 49.500 | 48.00 | 75.00 |
| 113 | Graphics | 800 x 600 | 32,768 | 100 x 37 | A0000 | 8 x 16 | 1 | 58.00 | 56.50 | 90.00 |
| 113 | Graphics | 800 x 600 | 32,768 | 100 x 37 | A0000 | 8 x 16 | 1 | 45.00*2 | 78.00 | 120.00 |
| 113 | Graphics | 800 x 600 | 32,768 | 100 x 37 | A0000 | 8 x 16 | 1 | 65.00*2 | 100.00 | 150.00 |
| 114 | Graphics | 800 x 600 | 65,536 | 100 x 37 | A0000 | 8 x 16 | 1 | 36.00 | 35.50 | 56.00 |
| 114 | Graphics | 800 x 600 | 65,536 | 100 x 37 | A0000 | 8 x 16 | 1 | 40.00 | 38.00 | 60.00 |
| 114 | Graphics | 800 x 600 | 65,536 | 100 x 37 | A0000 | 8 x 16 | 1 | 50.00 | 48.00 | 72.00 |
| 114 | Graphics | 800 x 600 | 65,536 | 100 x 37 | A0000 | 8 x 16 | 1 | 49.500 | 48.00 | 75.00 |
| 114 | Graphics | 800 x 600 | 65,536 | 100 x 37 | A0000 | 8 x 16 | 1 | 58.00 | 56.50 | 90.00 |
| 114 | Graphics | 800 x 600 | 65,536 | 100 x 37 | A0000 | 8 x 16 | 1 | 45.00*2 | 78.00 | 120.00 |
| 114 | Graphics | 800 x 600 | 65,536 | 100 x 37 | A0000 | 8 x 16 | 1 | 65.00*2 | 100.00 | 150.00 |
| 115 | Graphics | 800 x 600 | 16.7M | 100 x 37 | A0000 | 8 x 16 | 1 | 36.00 | 35.50 | 56.00 |
| 115 | Graphics | 800 x 600 | 16.7M | 100 x 37 | A0000 | 8 x 16 | 1 | 40.00 | 38.00 | 60.00 |
| 115 | Graphics | 800 x 600 | 16.7M | 100 x 37 | A0000 | 8 x 16 | 1 | 50.00 | 48.00 | 72.00 |
| 115 | Graphics | 800 x 600 | 16.7M | 100 x 37 | A0000 | 8 x 16 | 1 | 49.500 | 48.00 | 75.00 |
| 115 | Graphics | 800 x 600 | 16.7M | 100 x 37 | A0000 | 8 x 16 | 1 | 58.00 | 56.50 | 90.00 |
| 115 | Graphics | 800 x 600 | 16.7M | 100 x 37 | A0000 | 8 x 16 | 1 | 45.00*2 | 78.00 | 120.00 |
| 116 | Graphics | 1024 x 768 | 32,768 | 128 x 48 | A0000 | 8 x 16 | 1 | 65.00 | 48.00 | 60.00 |
| 116 | Graphics | 1024 x 768 | 32,768 | 128 x 48 | A0000 | 8 x 16 | 1 | 75.00 | 56.00 | 70.00 |
| 116 | Graphics | 1024 x 768 | 32,768 | 128 x 48 | A0000 | 8 x 16 | 1 | 80.00 | 60.00 | 75.00 |
| 116 | Graphics | 1024 x 768 | 32,768 | 128 x 48 | A0000 | 8 x 16 | 1 | 52.50*2 | 74.00 | 90.00 |
| 116 | Graphics | 1024 x 768 | 32,768 | 128 x 48 | A0000 | 8 x 16 | 1 | 85.00*2 | 100.00 | 120.00 |
| 117 | Graphics | 1024 x 768 | 65,536 | 128 x 48 | A0000 | 8 x 16 | 1 | 65.00 | 48.00 | 60.00 |
| 117 | Graphics | 1024 x 768 | 65,536 | 128 x 48 | A0000 | 8 x 16 | 1 | 75.00 | 56.00 | 70.00 |
| 117 | Graphics | 1024 x 768 | 65,536 | 128 x 48 | A0000 | 8 x 16 | 1 | 80.00 | 60.00 | 75.00 |
| 117 | Graphics | 1024 x 768 | 65,536 | 128 x 48 | A0000 | 8 x 16 | 1 | 52.50*2 | 74.00 | 90.00 |
| 117 | Graphics | 1024 x 768 | 65,536 | 128 x 48 | A0000 | 8 x 16 | 1 | 85.00*2 | 100.00 | 120.00 |
| 118 4 | Graphics | 1024 x 768 | 16.7M | 128 x 48 | A0000 | 8 x 16 | 1 | 65.00 | 48.00 | 60.00 |
| 118 4 | Graphics | 1024 x 768 | 16.7M | 128 x 48 | A0000 | 8 x 16 | 1 | 75.00 | 56.00 | 70.00 |
| 118 4 | Graphics | 1024 x 768 | 16.7M | 128 x 48 | A0000 | 8 x 16 | 1 | 80.00 | 60.00 | 75.00 |
| 118 4 | Graphics | 1024 x 768 | 16.7M | 128 x 48 | A0000 | 8 x 16 | 1 | 52.50*2 | 74.00 | 90.00 |
| 119 4 | Graphics | 1280 x 1024 | 32,768 | 160 x 64 | A0000 | 8 x 16 | 1 | 40.00*2 | 48.00 | 87.00(I) |
| 119 4 | Graphics | 1280 x 1024 | 32,768 | 160 x 64 | A0000 | 8 x 16 | 1 | 55.00*2 | 64.00 | 60.00 |
| 119 4 | Graphics | 1280 x 1024 | 32,768 | 160 x 64 | A0000 | 8 x 16 | 1 | 65.00*2 | 78.00 | 72.50 |
| 119 4 | Graphics | 1280 x 1024 | 32,768 | 160 x 64 | A0000 | 8 x 16 | 1 | 68.00*2 | 80.00 | 75.50 |
| 119 4 | Graphics | 1280 x 1024 | 32,768 | 160 x 64 | A0000 | 8 x 16 | 1 | 85.00*2 | 92.00 | 85.00 |
| 119 4 | Graphics | 1280 x 1024 | 32,768 | 160 x 64 | A0000 | 8 x 16 | 1 | 204.00 | 108.00 | 100.00 |
| 11A 4 | Graphics | 1280 x 1024 | 65,536 | 160 x 64 | A0000 | 8 x 16 | 1 | 40.00*2 | 48.00 | 87.00(I) |
| 11A 4 | Graphics | 1280 x 1024 | 65,536 | 160 x 64 | A0000 | 8 x 16 | 1 | 55.00*2 | 64.00 | 60.00 |
| 11A 4 | Graphics | 1280 x 1024 | 65,536 | 160 x 64 | A0000 | 8 x 16 | 1 | 65.00*2 | 78.00 | 72.50 |
| 11A 4 | Graphics | 1280 x 1024 | 65,536 | 160 x 64 | A0000 | 8 x 16 | 1 | 68.00*2 | 80.00 | 75.50 |
| 11A 4 | Graphics | 1280 x 1024 | 65,536 | 160 x 64 | A0000 | 8 x 16 | 1 | 85.00*2 | 92.00 | 85.00 |
| 11A 4 | Graphics | 1280 x 1024 | 65,536 | 160 x 64 | A0000 | 8 x 16 | 1 | 204.00 | 108.00 | 100.00 |
| 11B 4 | Graphics | 1280 x 1024 | 16.7M | 160 x 64 | A0000 | 8 x 16 | 1 | 80.00 | 48.00 | 87.00(I) |
| 11B 4 | Graphics | 1280 x 1024 | 16.7M | 160 x 64 | A0000 | 8 x 16 | 1 | 110.00 | 64.00 | 60.00 |
| 11B 4 | Graphics | 1280 x 1024 | 16.7M | 160 x 64 | A0000 | 8 x 16 | 1 | 130.00 | 78.00 | 72.50 |
| 11B 4 | Graphics | 1280 x 1024 | 16.7M | 160 x 64 | A0000 | 8 x 16 | 1 | 135.00 | 80.00 | 75.5 |
| 120 | Graphics | 1600 x 1200 | 256 | 160 x 64 | A0000 | 8 x 16 | 1 | 80.00*2 | 76.50 | 60.00 |
| 120 | Graphics | 1600 x 1200 | 256 | 160 x 64 | A0000 | 8 x 16 | 1 | 200.00 | 90.00 | 72.00 |
| 121 4 | Graphics | 1600 x 1200 | 32768 | 160 x 64 | A0000 | 8 x 16 | 1 | 80.00*2 | 76.50 | 60.00 |
| 121 4 | Graphics | 1600 x 1200 | 32768 | 160 x 64 | A0000 | 8 x 16 | 1 | 200.00 | 90.00 | 72.00 |
| 122 4 | Graphics | 1600 x 1200 | 65,536 | 160 x 64 | A0000 | 8 x 16 | 1 | 80.00*2 | 76.50 | 60.00 |
| 122 4 | Graphics | 1600 x 1200 | 65,536 | 160 x 64 | A0000 | 8 x 16 | 1 | 200.00 | 90.00 | 72.00 |
| 207 | Graphics | 1152x864 | 256 | 160 x 64 | A0000 | 8 x 16 | 1 | 80.00*2 | 55.00 | 60.00 |

Notes:

(I)	Interlaced modes

*	Extended text modes; 350 scan lines

**	9x16 character cell text modes; 400 scan lines

4	Available on 4MB Terminator models only