INSTL311.DOC

Kingston Technology Company

INSTALLATION GUIDE

Novell Netware Ver.3.11 Server LAN Driver

for

EtheRx PCI 10/100 Fast Ethernet Adapter (KNE110TX)

Copyright (C) 1996 Kingston Technology Company.

All rights reserved.

This installation guide provides the following instructions:

-  Using QStart to transfer Novell Netware Ver.3.11 Server LAN

Driver files from the Kingston Drivers Diskette to the

destination directory.

-  Installing the Novell Netware Ver.3.11 Server LAN Driver.

Driver Installation Procedure:

I. Extracting the driver files from the compressed file

When you use the Generic or Custom install options in QStart, the

files associated with the network operating system environment you

choose will be displayed on the screen and copied to the default

destination directory (C:\NETWARE for Netware 3.11), which is also

the default directory for the Netware 3.11 system files.

If you changed the default directory when you loaded Netware 3.11,

please make sure when you run QStart, you specify this same directory.

1.

Make sure the EtheRx PCI adapter is properly installed in

your computer.

2.

Insert the Kingston Drivers diskette into your floppy drive.

3.

Type A:QSTART[Enter] to run QStart.

4.

Select 'Generic Install', after the adapter test completed

select [Continue].

5.

Select 'Novell Netware 386 Ver.3.11 Server'.

6.

Change the default destination directory if desired then click

'Copy'.

7.

After QStart copied the files to the destination directory,

click 'Exit' to get back to DOS.

II. Installing Novell Server Driver

1.

Make sure the EtheRx PCI adapter is properly installed in

your computer.

2.

Start your Novell file server as you usually do.

3.

KTC110.LAN requires certain updated NLMs to be loaded

before loading itself on a NetWare 3.11 server.

The required NLMs are:

- LSLENH.NLM

- ETHERTSM.NLM

- MSM31X.NLM

- PATCHMAN.NLM

Note: Make sure these NLM modules are in the same directory.

To view statistics and information on the Server from the

MONITOR.NLM, you should update the MONITOR.NLM from

C:\NETWARE  for NetWare 3.11.

4.

At the system console screen, type the following command

(these commands are order sensitive):

: Load C:\NETWARE\PATCHMAN.NLM

: Load C:\NETWARE\LSLENH.NLM

: Load C:\NETWARE\KTC110 FRAME=<frame type> SLOT=1

Note:

SLOT=1 for 1st adapter, 2 for 2nd and so on

5.

Bind the driver to the IPX protocol by typing:

: Bind ipx to KTC110X NET=<network number>

6.

To load the driver automatically, you must add the following

command lines to your AUTOEXEC.NCF file.

Type 'LOAD INSTALL' at the server prompt to access the install

utility. Open the "SYSTEM OPTIONS" menu to access "EDIT

AUTOEXEC.NCF file" and enter the following command

into AUTOEXEC.NCF

Load C:\NETWARE\PATCHMAN

Load C:\NETWARE\LSLENH.NLM

Load C:\NETWARE\KTC110 FRAME=<frame type> SLOT=1

7.

The installation is now complete.

Kingston Technology, EtheRx, and QStart are registered

trademarks of Kingston Technology Company.  All others are

trademarks or registered trademarks of their respective holders.