README.DOC for Crystal Semiconductor Corp. 1.80 Windows 95 Drivers

This Document contains the following:

1. Version Numbers

2. List of files

3. File Summary

4. Installing Crystal Audio Drivers

5. Un-installing Crystal Audio Drivers

6. MS-DOS Mode

7. Multiple Crystal Audio Cards

8. Crystal 3D Audio - CW3DSND.EXE

1. Version Numbers:

1.70 		CS4232C.EXE

1.70b		CSFM.DRV

1.53 		CS32MIX.EXE

1.80d		CS32BA11.DRV

1.80d		CWAUDIO.VXD

1.70c		CS4232LD.VXD

1.51  	CSPCPLE.DLL

1.00.003	CW3DSND.EXE

1.59		SETUP.EXE

1.81		CW3DSND.EXE

Microcode:

79   		CS4232 microcode

6     	CS4232A microcode

55		CS4236 microcode

2. List of Files:

CSPCPLE	DLL	73,216  04-18-96  6:00p

WSTREAM	DLL	9,712  04-17-96  6:00p

CS32BA11	DRV	198,320  10-29-96  6:00p

CSFM		DRV	21,808  08-08-96  6:00p

CSMIDI		DRV	9,296  04-15-96  6:00p

CS32MIX	EXE	234,538  09-27-96  6:00p

CS4232C	EXE	89,696  09-26-96  6:00p

MSGLOOP	EXE	4,112  04-17-96  6:00p

SETUP		EXE	287,744  10-18-96  6:00p

CW3DSND	EXE	279,644  11-13-96  6:00p

CS4232		INI	774  10-02-95  6:00p

CS4232LD	VXD	13,380  08-09-96  6:00p

CSMIDI		VXD	28,844  05-02-96  6:00p

CWAUDIO	VXD	97,335  10-29-96  6:00p

VWSTREAM	VXD	5,175  04-18-96  6:00p

CWAUDIO	BIN	6,973  10-07-96  6:00p

CS4232		INF	22,709  11-13-96  6:00p

README	DOC

3. File Summary:

CS4232C.EXE   	CS4232/CS4236/CS4236B/CS4237B/CS4238b Configuration Program.  A DOS device driver used to initialize and configure the card.  It will download microcode from CWAUDIO.BIN.  Using the /X option, it will download resource data from RESOURCE.BIN, CS32RES.BIN, or CS36RES.BIN.

CS32MIX.EXE	DOS mixer used to set or adjust SoundBlaster mixer settings.  It will run as a GUI application or as a command line utility.

CWAUDIO.BIN  	Data file containing microcode for the CS4232, CS4232A, and CS4236.  It is used by CS4232LD.VXD and CS4232C.EXE.

CS4232.INI   	Default initialization parameters for CS4232C.EXE.  It is only used if PnP is overridden (/O) or if not using the Win 95 option on a non PnP system (no /W).

CS4232.INF  	Windows 95 installation file.  It will install the files, update the registry, install CS4232C.EXE /A in the CONFIG.SYS, and it will install CS4232C.EXE /O /R in the MS-DOS mode CONFIG.SYS.  It should be modified by OEM’s to meet their specific needs.

CSFM.DRV	Crystal FM MIDI driver.

CWAUDIO.VXD	CS4232/4236/4236B/4237B/4238B Windows 95 virtual device driver.

CS32BA11.DRV	CS4232/CS4236/CS4236B/CS4237B/CS4238B Windows 95 wave, auxiliary input, and mixer driver.

CS4232LD.VXD   	CS4232/CS4236/CS4236B/CS4237B/CS4238B Windows 95 Device Loader VxD.  This driver will download microcode from the CWAUDIO.BIN file.

CW3DSND.EXE		CS4237B/CS4238B 3D Audio Control Applet.

CSMIDI.VXD	Crystal MPU-401 MIDI virtual device driver.

CSMIDI.DRV	Crystal MPU-401 MIDI Driver.

CSPCPLE.DLL	Windows 95 Power Control Panel extension.  This file will create an extension to the default Windows 95 Power Control Panel, enabling new features like idle time-out and power-down now.

SETUP.EXE	Crystal Windows 95 un-install program.  Running this program will clean out any Crystal registry entries, delete Crystal files, clean the CONFIG.SYS, and prompt the user to restart.  When Windows 95 restarts, the Crystal drivers should install using the default Windows 95 installation.

VWTREAM.VXD		It is the main WaveStream engine.  It resides between

WSTREAM.DLL and CWAUDIO.VXD.  It does the Software 				WaveTable Synthesis, SoundBlaster, and mixing.	This 				VXD is provided in our kit to hook up with the 					Brooktree WaveStream CD.

WSTREAM.DLL		It is the WaveStream component which communicates with 			cs32ba11.drv and in turn passes information down to 				vwstream.vxd.  This DLL is provided in our kit to hook

up with the Brooktree WaveStream CD.

MSGLOOP.EXE		It is a WaveStream component which is provided in our 			kit to hook up with the Brooktree WaveStream CD.

4. Installing Crystal Audio Drivers

1.Turn PC off and insert audio card into an ISA slot unless audio device is present in your motherboard.

2.Turn PC on and start up Win95.

3.Win95 is a PnP OS.  It automatically recognizes new PnP audio hardware and creates a database for the hardware.  The following shows the order in which Win95 detects the Crystal logical devices:

A.Win95 detects the Crystal PnP Audio System Codec.  Win95 asks for the manufacturer's Disk.  Please select the appropriate drive.  If the Windows default driver is not grayed out, installing DirectSound games replaces Crystal drivers with Microsoft Windows Sound System drivers.

B.Win95 detects the Crystal PnP Audio System Joystick (Joystick in some platforms).

C.Win95 detects the Crystal PnP Audio System Control Registers (CS4232 Support Registers in some platforms.)

D.Win95 detects the Crystal PnP Audio System MPU-401 Compatible.

E.Win95 detects the Crystal PnP Audio System CD-ROM.  This device may not be found if the Plug and Play resources do not reference a CD.  If your audio device is not using the CD-ROM interface, this device will not be detected.

If any of the above (A through E) is not detected, a previous Win95 sound card installation was done.  For a clean reinstallation, the user should see the Un-installing Crystal Audio drivers section below.

5. Un-installing Crystal Audio Drivers

1.From Win95, run setup.exe provided in the Crystal installation diskette. The Setup will do an automatic cleaning up of the Win95 Crystal Audio registry entries. Many problems with reinstallation occur because of existing registry entries from previous installations and because of the \WINDOWS\INF\OEMx.INF files.  After running setup, the various CS4232 devices from the Control Panel should disappear.  It is recommended that prior to running Setup, remove the device entries in the Sound, Video, and Game Controller of the System Device Manager, and Crystal entries (if any) in the Other detected devices section of the Device Manager.

2.Shutdown and turn off your PC.

3.Turn PC on, Start Win95, Win95 will now detect new hardware, create an information database, and proceed with the installation of Crystal device drivers.

6. MS-DOS Mode

A.  Restarting the computer in MS-DOS mode.

Shutting down to MS-DOS mode from the Shutdown Win95 Menu will automatically run the CS4232C.EXE /A in dosstart.bat.  Therefore, shutting down to MS-DOS mode will automatically have the audio card configured for Dos games.

B.  Enabling support in MS-DOS prompt icon.

To enable real DOS mode with CS4232/CS4236/CS4236b/CS4237b/CS4238b support:

1.Select a DOS prompt icon.

2.Select Properties.

3.Go to the Program tab.

4.Select the Advanced... button.

5.Click MS-DOS Mode.

6.Click "Specify a new MS-DOS configuration."

7.The following line will be automatically placed into the new MS-DOS configuration (with right path):

DEVICE=C:\WIN95\CS4232C.EXE /A

7. Multiple Crystal Audio Cards

To enable a CS4232/CS4236 audio card in a system that

contains multiple Crystal audio cards:

1.  Add the following option to the CS4232C.EXE

entry in the CONFIG.SYS.

Device=C:\WINDOWS\CS4232C.EXE /W /Sxx

where xx is the OEM ID number for the CS4232

audio card to be enabled.

2.  The OEM ID number can be obtained from the

manufacturer.

8.  Crystal 3D Audio - CW3DSND.EXE

The 3D audio accessory allows you to control the 3D audio features of your audio system. The accessory runs each time you start your computer where it appears in the notification area of the task bar. The task bar is where the start button resides. The notification area is where the clock normally appears on the task bar.. The accessory is activated by clicking with the left mouse button on the logo in the notification area (If you have trouble finding the accessory, move the mouse over each icon in the notification area until you find the icon with a tool tip that says “Crystal 3D Audio”). You can also click on the logo with the right mouse button to see a menu with options for activating the control “Settings”, viewing the about box “About”, or exiting the accessory “Exit”.

Note: If for some reason the Crystal 3D Audio icon is missing, you can run it by choosing Programs | Accessories | Multimedia| Crystal 3D Audio Control from the Start menu.

Controlling your 3D audio

There are two sliders for controlling the sound of your 3D audio: on CS4237B systems they are labeled Center and Space, on CS4238B systems they are labeled Volume and Space. Moving the slider toward the plus sign increases the effect, moving the slider toward the minus sign decreases the effect. You may find that you prefer different settings for different types of music. You can save up to 3 configurations using the preset buttons. You can disable your 3D audio by un-checking the Enabled check box at the bottom of the accessory.

Preset Buttons

The 3D audio accessory provides three preset buttons, labeled 1, 2, and 3, that allow you to save your favorite 3D audio settings. To activate the settings click on the preset button with the left mouse button. To store your settings, change the description of the button that appears in the tool tip, or to restore the button to the factory defaults click with the right mouse button on the preset button to see the menu of options.