Windows Drivers for the Pro AudioSpectrum 16

Version 1.2

copyright 1991,1992, MEDIA VISION

(510) 770-8600

Thankyou for purchasing the Pro AudioSpectrum Plus or Pro

AudioSpectrum 16!  We think you're going to love your new card.   This

release (5/5/92) includes new Windows drivers for the Pro AudioSpectrum

family of audio adaptors.

These drivers require Windows 3.1 and will not work with Windows 3.0 with

Multimedia Extenstions.  Since Windows 3.1 now contains most of the features

of Multimedia Windows, we will assume that most of you will be upgrading

to Windows 3.1.

Media Vision will continue to improve and enhance the Windows drivers in

order to continue to be the best source of Multimedia Windows

products for your PC.  The new Pro AudioSpectrum cards are capable of so

much, we will probably be adding features to the drivers for another

year.

We will be working on supporting multiple Pro AudioSpectra in a

single system.  (Pro AudioSpectra is the plural form instead of the

awkward form 'Pro AudioSpectrums') Those of you that have upgraded from

the original Pro AudioSpectrum, don't sell your old one just yet.  With

two Pro AudioSpectra you might want to record on one while playing back

from the other.  Supporting multiple cards in the system requires new ways

of handling multiple mixers, synthesizers, etc.  When we sort all this

out, you can expect new drivers.

If you want to stay current with what's happening in the world of

multimedia, windows, and the Pro AudioSpectrum 16, subscribe to

CompuServe.  Once online type GO MEDIAVISION.  This will take you to

the Media Vision section of the Multimedia Vendor forum.  Each day,

Monday thru Friday, a Media Vision representative will stop by to

answer your questions.  We also hope to establish a presence on

Internet, BIX, America Online and Prodigy.  Look for our

representatives on those services.

Be sure to check the NEWS section toward the end of this file.

IMPORTANT:  THE SND BLASTER SECTION OF THE PRO AUDIOSPECTRUM 16 IS, BY

DEFAULT, SET TO IRQ 7, DMA 1, PORT 220.  DO NOT ATTEMPT TO CONFIGURE

THE PROAUDIOSPECTRUM TO THE SAME IRQ OR DMA CHANNEL AS THE SND

BLASTER.  IF YOU DO, THE SYMPTOM WILL BE A SHORT SECTION OF AUDIO

PLAYED OVER AND OVER AND NOT STOPPING.  SHOULD YOU HEAR UNENDING SHORT

LOOPS OF SOUND , CHECK FOR IRQ CONFLICTS.

Here is a description of the files on this disk:

OEMSETUP.INF    This file allows you to add drivers to Windows using

the drivers applet in the control panel.  If you

use the drivers applet, you will not have to

manually edit your system.ini file.  Using the

Install disk will, in most cases be preferable to

installing drivers via the drivers applet.

MIX.EXE         This program allows you to modify those mixing controls

that you'll want to change most often.

Its main difference from promix is that it doesn't

cover nearly as much screen area.

This program can be added to your \windows directory.

PROMIX.EXE      ProMix can be launced from the WIN.INI 'load' line.

If you have Windows 3.1, use the Program Manager's

File/Preferences menu item to select the open

minimized option.

This program can be added to your \windows directory.

Here's a nifty feature:  When running Windows, you can

use the CTRL-ALT-U, CTRL-ALT-D, and CTRL-ALT-M sequences

to adjust the master volume.

The system menu contains a new feature: a pair of

DB meters.  The DB meters are active only during record

and playback of waveform data.

Owners of the original Pro AudioSpectrum may be

surprised to see two new sliders on the PROMIXER.

The BLSTR channel controls the Snd Blaster-compatible

digital audio source.  The new slider on the far

right is a master record level that lets you adjust

the total volume of all sources routed to the record

mixer.

MMMIXER.DLL     This DLL must be in your \WINDOWS\SYSTEM directory.

MMMIXER.DLL is like MMSYSTEM.DLL in a couple of ways.

First, it provides a driver-independent layer for all

mixer API's.  Second, it loads any mixer device drivers

in the system.

If you build an application that makes use of our

mixer API's, this DLL handles the call.  If there are

no mixer device drivers in the system, the mixer

API's will simply return an error.  This means you

can utilize the Pro AudioSpectrum 16's mixer capabilities

without requiring the user to have our hardware.

No changes this release.

The mixer DLL has two BIG selling points:

1) Interrupt-driven timed volume fades

If you're a programmer, investigate

the mixSetState function as defined

in the MIXSPEC directory.

2) Startup volume initialization

The first time the MIXER driver is loaded

it reads WIN.INI for volume setting and

patch information.  When Windows shuts

down, the current settings are saved.

Here's a sample WIN.INI entry:

/////////////////////////////////////////////////////

[mvmixer.drv]

LineIn1=SYNTH     MUS]  PLAY     L=100% R=100%  @=0000

LineIn2=MIXER     MIX]  PLAY     L= 72% R= 72%  @=0000

LineIn3=AUX       AUX|  PLAY     L=100% R=100%  @=0000

LineIn4=CD        CDA|  PLAY     L=  0% R=  0%  @=0000

LineIn5=MIC       MIC|  PLAY     L=  0% R=  0%  @=0000

LineIn6=DIGITAL   WAV]  RECORD   L=  0% R=  0%  @=0000

LineIn7=SPKR      SPK]  PLAY     L=100% R=100%  @=0000

LineIn7=BLSTR     WAV]  PLAY     L=100% R=100%  @=0000

LineOut1=MASTER   AMP|  PLAY     L= 79% R= 79%  @=0000

LineOut2=RECORD   WAV|  PLAY     L=100% R=200%  @=0000

StereoEnhance=ON

Loudness=ON

Bass=50

Treble=78

SaveSettingsOnWindowsExit=YES

LINE FIELD:

There are 7 input Lines and 1 output line.

Note: the Pro AudioSpectrum 16 has a second output

line.

PATCH NAME:

The field immediately after the '=' is the patch name.

These names (ie. SYNTH) appear in promix.exe at the

bottom of each slider pair.

DEVICE TYPE

The next field is the device type field.  The device

type determines the type of icon displayed in MIX.EXE

Each device type has a standard 3-letter mnemonic name

that can be obtained by calling mixGetDeviceName.  The

current list of types is:

N/C   No Connection             SPK        PC speaker

CDA   Compact Disk Player       AAT        Analog Audio Tape

AUX   Auxillary                 DAT        Digital Audio Tape

MIC   Microphone                TEL        Telephone

VTR   Video Tape Recorder       MIX        Mixer

VDP   Video Disc Player         EQU        Equalizer

MUS   Musical Instrument        AMP        Amplifier

WAV   Digital Waveform Player   LPA        Phonograph Audio

RAD   Radio                     TVA        Television Audio

UNK   Unknown Device Type

The fourth character indicates whether the user is

responsible for making the physical device connection.

Think of the character as the mounting bracket of an audio

card.  A hole, the '|' character, represents a mini

phone jack.  If the fourth character is ']', the connection

is made by the driver.  The '|' character indicates a jack

or connector must be hooked up before audio can be

heard.  User-friendly applications may use this information

to remind the user to connect a particular device.

OUTPUT CONNECTION

The next field corresponds to the internal mixer

connection info.  The word PLAY indicates that input

is routed to the PLAY output (LineOut1).  RECORD

means the input is routed to the ADC circuitry.

VOLUMES

If it isn't obvious, the L= and R= fields specify the

volumes of the left and right channels at start-up.

ASSOCIATION

This field is used to save information that specifies

a relationship between a mixer input and some other

device in the system.  For example, if there are two

CD's in the system and one is connected to LineIn4

and the other is connected to LineIn3, the

dwAssociation can be used to determine which MCI CD

device is on which input.

MISC

The other lines are self explanatory.  In the future

we will let users change the settings through control

panel's driver setup.

CHANGING WIN.INI

As an example, if you connect your Walkman to LineIn3

(AUX), you can replace the word AUX with RADIO.

Then, change the device type field to RAD|

/////////////////////////////////////////////////////

This DLL was developed to provide MultiMedia developers

better control over audio mixing functionality than

is afforded by the current volume API's.  Every

attempt was made in the API design to avoid its being

biased toward any specific mixer hardware.

Furthermore, the driver design is modeled after Microsoft's

Multimedia Windows driver design in order to facilitate

its acceptance.

We are offering this DLL to Microsoft and the MPC

community as a proposed standard for Multimedia Windows

volume control.  There is no license required to use

MMMIXER.DLL and you may freely distribute it with your

products.

Hardware manufacturers are encouraged to develop there own

drivers to work with MMMIXER.DLL.  Media Vision does

not yet offer a DDK for mixer drivers.  Parties

interested in the source to mvmixer.drv should

contact Media Vision's Jim Gifford at (415) 770-8600.

See the MIXSPEC directory for detailed information on the

Multimedia Mixer API.

MVMIXER.DRV     This file belongs in your \WINDOWS\SYSTEM directory.

This is Media Vision's Mixer driver for the Pro

AudioSpectrum family of products.  In the [drivers]

section of your system.ini file, you must have the line:

mixer=mvmixer.drv

MCIMIXER.DRV    Put this file in your \WINDOWS\SYSTEM directory.

This driver provides a set of extended MCI commands

for accessing mixer capabiities.

Under the [mci] section of your system.ini file, you

must have the line:

mixer=mcimixer.drv

Developers may ship this driver, license free, with

their products.

See the MIXSPEC directory for detailed information on the

Multimedia Mixer MCI command set (MCIMIXER.DOC).

MVPROAUD.DRV    This file belongs in your \WINDOWS\SYSTEM directory.

This driver supports WAVE (digital sound) audio, MIDI I/O

and auxillary volume control.

In the [drivers] section of your system.ini file, you

should have the following three entries:

wave=mvproaud.drv

midi=mvproaud.drv

aux=mvproaud.drv

This driver keeps configuration information in your

system.ini file under the header [mvproaud.drv]

If it fails to find that entry, it will

check to see if MVSOUND.SYS is loaded and use it's

settings.  Failing that, it will look for the

outmoded [mvwave.drv] and [mvmidi.drv] entries.

As a last resort, the driver will default to IRQ=7

and DMA=1.

When our aux driver is the only one in the drivers

section, a call to auxGetNumDevs returns a value

indicating 4 aux devices are in the system:

Device 0 is internal CD-ROM volume

Device 1 is auxillary (line in) volume

Device 2 is microphone volume

Device 3 is master volume

While we provide a Multimedia Mixer library, we still

support the existing volume control calls, auxGetVolume

and auxSetVolume, when this driver is installed.

In the [drivers] section of your system.ini file, you

must add the line:

aux=mvaux.drv

VPASD.386       This file belongs in your \WINDOWS\SYSTEM directory.

It provides extra buffer space needed to handle

16-bit digital audio.

You must have the following line in system.ini's

[386Enh] section:

device=vpasd.386

Also you must add the following line to system.ini's

[mvproaud.drv] section:

buffer=32

TBWAVE.DRV      This is the .WAV driver for Media Vision's Snd

Blaster-compatible ThunderBoard product.  It can

be added to your system.ini file giving you a second

digital audio device:

wave1=tbwave.drv

This driver is not automatically installed.  Many

low-end PC's may not be able to accomodate two

digital audio channels playing simultaneously.

CPU speed, hard disk speed and available RAM will

determine whether your system can handle dual DACs

and at what sampling rates.

VTBD.386        Put this file in the \WINDOWS\SYSTEM directory if you

will be using tbwave.drv

You must have the following line in system.ini's

[386Enh] section:

device=vtbd.386

VADMAD.386      Put this file in the \WINDOWS\SYSTEM directory.

This file is not required for Windows 3.1.

VADMAD stands for "virtual audio DMA device".  If you

run Windows in 386 enhanced mode, you must add

device=vadmad.386

to the [386Enh] section of system.ini.  This device

driver allows our wave driver to use auto-initialized

DMA mode.  When this file is not present, you will

hear a PC-speaker beep as Windows boots and you will

not be able to play any digital audio sounds.

MCIMIXER.DOC    These two files describe Media Vision's proposed

MIXAPI.DOC      standard for software controlled mixer devices.

_____________________________________________________________________

SAMPLE SYSTEM.INI FILE DRIVERS SECTION

[drivers]

timer=timer.drv

Wave=mvproaud.drv

Wave1=tbwave.drv

MIDI=mvproaud.drv

MIDI1=opl3.drv

mixer=mvmixer.drv

Aux=mvproaud.drv

[mci]

CDAudio=mcicda.drv  **

WaveAudio=mciwave.drv 4

Sequencer=mciseq.drv

mixer=mcimixer.drv

** Windows 3.1 owners.  The mcicda.drv can be installed if you have

a MPC-compatible CD-ROM drive.  This driver will allow you to control

your CD through MCI programs like MPLAYER.  MCICDA.DRV can be found

on one of your Microsoft Windows installation diskettes.  The file

may be compressed.  If so, it will have an underscore as the last

character.  You can decompress it using the expand utility, and type

EXPAND mcicda.dr_ c:\windows\system\mcicda.drv.

_____________________________________________________________________

MEDIA VISION NEWS:

Media Vision will shortly announce two new audio adaptors:

The Pro AudioSpectrum Plus with Sony interface  AND

The Pro AudioSpectrum 16   with Sony interface

This means that owners of Sony's Laser Library or other CD-ROM

drive's that use the Sony bus will be able to upgrade their system

to an MPC by getting one of the above cards.

Installation Notes

------------------

The sub-mini jacks at the back of the board are:

Top:    External Input

Middle: Amplified Output

Bottom: Microphone input (MONO)

The four pin header connector is for the PC-speaker input.

The five pin header connector is for the CD-Audio internal input.

There is a second five pin header in the center of the Pro-16.

This header is for a second internal CD input.  That input will be

supported in the next software release.  That input will take over

the so-called "record monitor" input which appears as the second

channel from the left on the Pro Mixer (PROMIX.EXE)

If you have bugs to report, technical questions or feature requests

regarding this software, please contact Media Vision Technical Support:

Tech Support Voice: (800) 638-2807 or (510) 770-9905

Tech support's FAX: (510) 770-9592.

BBS: (510) 770-0968   2400b  8n1

BBS: (510) 770-1661   2400b  8n1

BBS: (510) 770-0527   9600b  8n1

We will shortly be adding more BBS lines and more tech support lines.

#